COA Module 4 — Embedded I/O Systems

GPIO · Serial I/O (SPI & UART) · Other Peripherals

# 1. What is an Embedded I/O System?

Simple answer: It is how a microcontroller talks to the physical world — reading sensors/switches (input) and controlling motors/LEDs/displays (output).

Think of a washing machine: the MCU inside reads the dial and water level sensor (input), then controls the motor and water valve (output). That communication system is the embedded I/O system.

Analogy: Human nervous system Your brain (MCU) receives signals from eyes, ears, skin (sensors = inputs) and sends commands to muscles (actuators = outputs). The nerves are the I/O system.

## 1.1 Architecture

| Input Devices | Embedded Processor | Output Devices |
| --- | --- | --- |
| Sensors (temp, pressure) | Microcontroller (MCU) | Actuators (motors, valves) |
| Switches, Keypads | Memory | LCD Displays, LEDs |
| Cameras, Microphones | I/O Ports | Buzzers, Relays, Motors |

## 1.2 What is a Microcontroller (MCU)?

One chip: Processor + Memory + I/O peripherals. A complete tiny computer.

- Cost: Few cents to a few dollars.

- Power: Milliwatts — runs on small battery.

- Everywhere: Microwave, TV remote, car ECU, smartwatch, traffic lights.

## 1.3 RED-V Board (Real-world MCU example)

RED-V is a RISC-V development board built around the SiFive FE310 SoC.

| Feature | Detail |
| --- | --- |
| Processor | 32-bit RISC-V (RV32IMAC), 5-stage pipeline |
| Speed | 320+ MHz |
| I/O Pins | 19 digital GPIO, 9 PWM pins (Arduino footprint) |
| Memory | 16KB instruction cache + 16KB data scratchpad |
| Connectivity | USB-C for programming, Qwiic for I2C devices |

# 2. GPIO — General Purpose Input/Output

## 2.1 What is GPIO?

GPIO pins are the most basic I/O on any MCU. Simple digital pins — configure as input or output in software.

- Output mode: Write 1 → pin HIGH (3.3V). Write 0 → pin LOW (0V). Turns on LEDs, drives buzzers.

- Input mode: Read the pin. Switch closed → reads 1. Switch open → reads 0.

Analogy: Light switch GPIO is like a light switch. You can flip it on/off (output mode), or just check if someone else flipped it (input mode).

## 2.2 Hardware Concepts

Pull-down resistor (1kOhm): When a switch is open, the pin floats (undefined). The pull-down connects pin to GND, forcing it to read 0. Closing the switch pulls it to 3.3V, reads 1.

Current-limiting resistor (330 Ohm) for LEDs: GPIO outputs only ~16mA. Without this resistor, the LED burns out and damages the GPIO pin.

GPIO gives ~16mA. DC motors need ~1A. NEVER connect a motor directly to GPIO — use an H-bridge driver circuit.

## 2.3 GPIO Registers (Memory-Mapped I/O)

The MCU controls GPIO through special memory addresses called registers. Each bit = one GPIO pin.

| Offset | Register | What it does |
| --- | --- | --- |
| 0x00 | input_val | Read current values of all input pins |
| 0x04 | input_en | Set bit=1 to enable pin as INPUT |
| 0x08 | output_en | Set bit=1 to enable pin as OUTPUT |
| 0x0C | output_val | Write 1 or 0 to control output pins |
| 0x10 | pue | Enable internal pull-up resistor |
| 0x18 | rise_ie | Enable interrupt when pin goes 0 to 1 (button press) |
| 0x20 | fall_ie | Enable interrupt when pin goes 1 to 0 (button release) |
| 0x38 | iof_en | Set = hardware owns pin. Clear = use as GPIO |

Setup sequence: (1) Clear iof_en to take pin from hardware. (2) Set input_en OR output_en. (3) Then read input_val or write output_val.

## 2.4 GPIO Interrupts

Configure a GPIO pin to fire an interrupt automatically on events — no polling needed:

- rise_ie: Fires when pin goes 0 to 1 (button pressed).

- fall_ie: Fires when pin goes 1 to 0 (button released).

- high_ie / low_ie: Fires while pin stays HIGH or LOW.

# 3. Serial I/O — Sending Data Bit by Bit

## 3.1 Serial vs Parallel

|  | Parallel I/O | Serial I/O |
| --- | --- | --- |
| Method | All 8 bits sent on 8 wires simultaneously | 1 bit at a time on 1 wire |
| Wires needed | 8+ wires for 1 byte | 1 to 4 wires |
| Speed | Faster per byte | Slower but good enough |
| Pin usage | Wastes many MCU pins | Saves pins |

## 3.2 SPI — Synchronous Serial

Synchronous: Both devices share a clock signal (SCK). Controller generates it.

| Pin | Name | Purpose |
| --- | --- | --- |
| SCK | Serial Clock | Controller generates clock. Both sync to this. |
| SDO | Serial Data Out | Sends data OUT from this device. |
| SDI | Serial Data In | Receives data INTO this device. |
| CE | Chip Enable | Active-LOW. Pulled LOW to select a peripheral. |

### How SPI sends 1 byte — step by step

- Controller pulls CE LOW — selects the peripheral, signals transmission starting.

- Controller starts clock pulses on SCK.

- Data changes on the FALLING edge of SCK (sender puts bit on the line).

- Data is sampled on the RISING edge of SCK (receiver reads the bit).

- MSB (bit 7) is sent first, down to bit 0 — all 8 bits in 8 clock cycles.

- Simultaneously the peripheral sends back 8 bits — full duplex!

- CE goes HIGH — transmission complete.

SPI is full-duplex — while MCU sends 8 bits to a sensor, the sensor sends 8 bits back in the same 8 clock cycles. Two-way simultaneous transfer.

- Pros: Fast, full-duplex, simple.

- Cons: Needs one CE pin per device — many devices = many wires.

## 3.3 UART — Asynchronous Serial

Asynchronous: No shared clock. Both devices independently run at the same agreed speed called the baud rate.

Analogy: Letters vs Phone call SPI = phone call (synchronized real-time). UART = exchanging letters — no live clock, but both agree on timing rules so messages are understood.

### UART Frame — how every byte is packaged

| Part | Value | Why it exists |
| --- | --- | --- |
| Idle | Line = 1 | When nothing is sent, line stays HIGH. |
| Start bit | Always 0 | Falling edge (1 to 0) tells receiver: a byte is coming! Receiver locks on. |
| Data bits | 7 or 8 bits | The actual byte, usually 8 bits, LSB first. |
| Parity bit | Optional | Error detection bit. Rarely used today. |
| Stop bit | Always 1 | Signals end of byte. Line returns to idle. |

Example sending A (ASCII 65): [START: 0] [1 0 0 0 0 0 1 0] [STOP: 1]. Receiver finds byte boundaries from start/stop bits — no clock wire needed!

### TX and RX — the cross connection rule

TX (Transmit): Pin that sends data OUT.

RX (Receive): Pin that receives data IN.

Golden rule: TX of Device 1 MUST connect to RX of Device 2, and vice versa. Always cross!

TX to TX is the most common beginner mistake. What one device SENDS, the other must RECEIVE. So TX goes to RX.

### SPI vs UART

| Feature | SPI | UART |
| --- | --- | --- |
| Clock | Shared (synchronous) | No shared clock (async) |
| Wires | 3 to 4 (SCK+SDO+SDI+CE) | 2 (TX + RX) |
| Speed | Faster | Slower |
| Full duplex | Yes | Yes |
| Used for | Sensors, Flash memory | GPS, PC comms |

# 4. Other Peripherals

## 4.1 Character LCD

Displays simple text — like the screen on a microwave or ATM.

- How: MCU sends ASCII codes. LCD built-in controller maps codes to pixels and lights up the display.

- Interface: Parallel, SPI, or RS-232. Very easy to add.

- Used in: Cash registers, printers, fax machines, vending machines.

## 4.2 VGA Monitor

VGA (1987): 640x480 pixels. 15-pin connector. Still backward compatible with modern monitors.

### How CRT display works (basis of VGA timing)

- Electron gun: Fires electrons scanning left-to-right, line by line.

- Phosphors: 3 per pixel (Red, Green, Blue). Beam hits them, they glow. Beam strength = color intensity.

- Horizontal blanking: After each line, gun turns OFF and returns to start of next line.

- Vertical blanking: After all lines, gun turns OFF and returns to top-left corner.

- Refresh rate: 60-75 times per second. Eye sees a steady image.

| VGA Parameter | Value | Meaning |
| --- | --- | --- |
| Total scan lines | 525 | Full frame height including blanking |
| Active scan lines | 480 | Lines that actually show image |
| Total pixels per line | 800 | Full line width including blanking |
| Active pixels per line | 640 | Pixels that show actual image |
| Remaining 45 lines + 160 pixels | Non-active | Black — used for sync timing |

Modern LCDs do not use electron guns, but they use the same VGA timing signals for backward compatibility with older systems.

## 4.3 Bluetooth (BlueSMiRF)

Wireless replacement for a UART cable. MCU talks to Bluetooth module via UART; module wirelessly relays data to laptop or phone.

- Band: 2.4 GHz ISM — unlicensed, anyone can use it.

- Channels: 79 channels, each 1 MHz wide, starting at 2402 MHz.

- Frequency hopping: Jumps channels ~1600 times/second in pseudorandom pattern — avoids sustained interference with Wi-Fi routers in the same band.

Why frequency hopping? Bluetooth and Wi-Fi share the 2.4 GHz band. By hopping channels in a pattern both paired devices know, Bluetooth avoids staying on any one channel long enough to cause persistent interference.

## 4.4 Motor Control

### DC Motor

- Works: Apply voltage to spin. Reverse voltage to reverse direction. Vary voltage to vary speed.

- Problem: Needs ~1A current. GPIO only gives ~16mA. Direct connection destroys the MCU.

- Solution — H-Bridge: Driver circuit between GPIO and motor. Amplifies small GPIO signal to drive high motor current. Enables direction control.

- Shaft encoder: Needed to track position or speed — sends pulses per revolution.

### Servo Motor

- Works: Controlled by PWM (Pulse Width Modulation). Vary pulse width to set exact angle (0 to 180 degrees).

- Easy: Built-in driver — just send PWM signal directly. No H-bridge needed.

- Limitation: Limited rotation range. Less powerful. Not for continuous spinning.

- Used for: Robot arms, RC car steering, camera gimbals.

### Stepper Motor

- Works: One pulse = one step (fixed angle, e.g. 1.8 degrees). Send 200 pulses = exactly 360 degrees rotation.

- Precise: Count pulses to know exact position — no encoder needed.

- Still needs H-bridge: Same high current problem as DC motor.

- Used for: 3D printers, CNC machines, precision positioning.

| Feature | DC Motor | Servo Motor | Stepper Motor |
| --- | --- | --- | --- |
| Control | Voltage / PWM | PWM pulse width | Pulse count |
| Position tracking | Needs encoder | Built-in (limited) | Count pulses |
| Rotation | Continuous 360 deg | Limited angle | Full 360 precise |
| Needs H-bridge? | YES | No | YES |
| Cost | Low | Medium | High |
| Best for | Fans, wheels | Arms, RC steering | 3D printers, CNC |

# 5. Quick Revision Summary

| Topic | Key Point |
| --- | --- |
| Embedded I/O | MCU reads sensors (input) + drives actuators (output). One cheap chip. |
| GPIO | Digital pin = 1 or 0. Configure via registers. Pull-down for switches, series resistor for LEDs. |
| Memory-mapped GPIO | Control pins by writing to memory offsets: input_val, output_en, iof_en, etc. |
| SPI | Synchronous. SCK+SDO+SDI+CE. Change on falling edge, sample on rising. Full-duplex. |
| UART | Asynchronous. No clock. TX to RX cross-connect. Frame: Start(0) + 8 bits + Stop(1). |
| Baud rate | Agreed UART speed. Both sides must match exactly. |
| Character LCD | Displays text. MCU sends ASCII codes. |
| VGA | 640x480 active pixels. 525 total scan lines (45 are blanking/sync). |
| Bluetooth | Wireless UART. 2.4 GHz, 79 channels, frequency hops to avoid Wi-Fi interference. |
| DC Motor | High current needs H-bridge. Continuous rotation. |
| Servo Motor | PWM controlled. Limited angle. No H-bridge needed. |
| Stepper Motor | One pulse = one precise step. Needs H-bridge. |

— End of Notes —